Division 1 General Requirements

Temporary Facilities

Section 01500

PART 1 - GENERAL

1.1  	Related Documents:

Drawings and general provisions of Contract, including General Conditions, Section 00700,and Supplemental General Conditions, Section 00810, and Division 1 requirements.

PART 2 - PRODUCTS  (NOT USED)

PART 3 - EXECUTION

3.1  	Office at the Work Site (Job Shack)

During the performance of this Contract, CONTRACTOR shall maintain a suitable office at or near the site of the Work which shall be the headquarters of his superintendent.  Any communication given to the superintendent or delivered to CONTRACTOR's office at the site of the Work in his absence shall be deemed to have been delivered to CONTRACTOR.

Delete the following paragraph if the construction schedule is less than six (6) months.

In addition, Contractor shall provide a suitable field office with at least 200 square feet of floor space, either adjacent to, or partitioned off from, his office at the site for use by Owner’s Representative. The office shall be provided with outside entrance door with a substantial lock, glazed windows suitable for light and ventilation, and adequate heating, air conditioning, and lighting facilities.  Contractor shall pay all electricity and heating bills and shall provide telephone services with a telephone as specified hereinafter.  The office shall be furnished with a desk, two four-drawer filing cabinets, a table, two chairs, a plan rack, and a locker for storage of surveying instruments.  The doors on the locker shall be equipped for padlocking.  The general arrangement of the office and facilities provided shall be acceptable to Owner’s Representative.

3.2  	Water for Construction

All water required for and in connection with the Work to be performed shall be furnished by and at the expense of the CONTRACTOR through meters installed on hydrants, except for water used in the “disinfection of potable water lines” process per Specification 510.3(29). All water used in the disinfection process shall not be metered, but rather shall be measured by calculation.  Such water use does not require a meter, but a double-check valve assembly is required when connecting to a fire hydrant or a City main.  CONTRACTOR shall submit a written plan for the disinfection process for review and approval by OWNER prior to commencing Work.  The written plan shall include the CONTRACTOR’s plan for final flushing and discharge of chlorinated water, and shall specify the quantities of potable water that will be required for the procedure and dosage plan proposed by the CONTRACTOR.  Water used in the disinfection process shall be supplied by the OWNER through hydrants or connection through a City main at no charge to the CONTRACTOR for the initial disinfection procedure up to the quantities agreed to in the written plan for the disinfection process.  Should the initial disinfection procedure fail to produce acceptable bacteriological sample test results, the cost of water at standard rates used for subsequent disinfection procedures shall be the responsibility of the CONTRACTOR with quantities determined through calculations.

For all water required for and in connection with the Work to be performed other than for the disinfection process, water and meters will be available from OWNER at standard rates. All costs for obtaining a water meter shall be the responsibility of the CONTRACTOR.  The CONTRACTOR shall contact the Austin Water Utility and arrange to pick up the meter.  CONTRACTOR shall install a  double-check valve assembly on the fire hydrant between the hydrant and the meter, to prevent backflow in the event of pressure failure.  CONTRACTOR shall supply all necessary tools, hose and pipe, and shall make necessary arrangements for securing and transporting such water and shall take water in such a manner, and at such times, that will not produce a harmful drain or decrease of pressure in the OWNER's water system.  It shall be the CONTRACTOR's responsibility to make arrangements with the Austin Water Utility for the metering and reporting of the amount of water used. Water shall not be used in a wasteful manner.  Standard hydrant wrenches shall be used for opening and closing of fire hydrants.  In no case shall pipe wrenches be used for this purpose.  Temporary lines shall be removed when no longer required.

“If applicable, after the installation of the City of Austin’s water meter(s) for the Project, the CONTRACTOR has the option to utilize the water available from this service at the CONTRACTOR’s expense.  An adjustment to the Contract amount will be made by Change Order at the end of the Project for the costs incurred by the City of Austin for the water.”

3.3  	Telephone Service

Contractor shall make all necessary arrangements and pay all installation charges for telephone lines in his offices at the site and shall provide all telephone instruments.  The telephone service shall be available to the Owner’s Representative for toll free calls.

3.4  	Sanitary Facilities

CONTRACTOR shall furnish temporary sanitary facilities at the site, as provided herein, for the needs of all construction workers and others performing Work or furnishing services on the Project.

Sanitary facilities shall be of reasonable capacity, properly maintained throughout the construction period, and obscured from public view to the greatest practical extent.  If toilets of the chemically treated type are used, at least one toilet shall be furnished for each 20 employees.  Contractor shall enforce the use of such sanitary facilities by all personnel at the site.

3.5  	Protection of Public and Private Property

Contractor shall protect, shore, brace, support and maintain all underground pipes, conduits, drains, and other underground construction uncovered or otherwise affected by the CONTRACTOR’s operations.  All pavement, surfacing, driveways, curbs, walks, buildings, utility poles, guy wires, fences, and other surface structures affected by construction operations, together with all sod and shrubs in yards, parkways, and medians, shall be restored to their original condition, whether within or outside the easement/right-of-way.  All replacements shall be made with new materials.

Contractor shall be responsible for all damage to streets, roads, curbs, sidewalks, highways, shoulders, ditches, embankments, culverts, bridges, or other public or private property, which may be caused by transporting equipment, materials, or men to or from the Work, whether by him or his Subcontractors. Contractor shall make satisfactory and acceptable arrangements with the owner of, or the agency having jurisdiction over, the damaged property concerning its repair or replacement or payment of costs incurred in connection with the damage.

All fire hydrants and water control valves shall be kept free from obstruction and available for use at all times.

3.6  	Tree and Plant Protection

All trees and other vegetation which must be removed to perform the Work shall be removed and disposed of by the CONTRACTOR; however, no trees or cultured plants shall be unnecessarily removed unless their removal is indicated on the Drawings.  All trees and plants not removed shall be protected against injury from construction operations.

No tree shall be removed outside of permanent easement(s), except where authorized by the E/A.  Whenever practicable, CONTRACTOR shall tunnel beneath trees in yards and parking lots when on or near the line of trenching operations.  Hand excavations shall be employed as necessary to prevent injury to trees. Care shall be taken with exposed roots, unearthed during construction, so that roots do not dehydrate causing tree damage.

Trees considered by the E/A to have any significant effect on construction operations are indicated on the Drawings and those which are to be preserved are so indicated.

CONTRACTOR shall take extra measures to protect trees designated to be preserved, using methods shown on the Drawings and as specified in Standard Specification Item No. 610S “Preservation of Trees and other Vegetation".

3.7   	Security

CONTRACTOR shall be responsible for protection of the site, and all Work, materials, equipment, and existing facilities hereon, against vandals and other unauthorized persons.

No claim shall be made against Owner by reason of any act of an employee or trespasser, and Contractor shall make good all damage to the Owner's property resulting from CONTRACTOR's failure to provide security measures as specified.

Security measures shall be at least equal to those usually provided by OWNER to protect existing facilities during normal operations, and shall also include such additional security fencing, barricades, lighting, and other measures as required to protect the site.  When required, the CONTRACTOR shall provide a security plan to the OWNER for review as to appropriateness of the security measures proposed.

3.8  	Access Roads

CONTRACTOR shall establish and maintain temporary access roads to various parts of the site as required to complete the Project.  Such roads shall be available for the use of all others performing Work or furnishing services in connection with the Project.

3.9  	Parking

Contractor shall provide and maintain suitable parking areas for the use of all construction workers and others performing Work or furnishing services in connection with the Project, as required, to avoid any need for parking personal vehicles where they may interfere with public traffic, the Owner's operations, or construction activities.

3.10   Dust Control

Dust Control during construction of this Project shall conform to Standard Specifications Item No. 220S, “Sprinkling for Dust Control".  No direct payment will be made for dust control.  Dust Control shall be considered subsidiary work relating to various Bid items of the Contract.

3.11   Temporary Drainage Provisions

Contractor shall be responsible for providing for the drainage of stormwater and such water as may be applied or discharged on the site in performance of the Work.  CONTRACTOR shall obtain E/A approval for temporary drainage facilities which will handle, carry through, or divert around his Work all drainage flow, including storm flow and flows created by construction activity, to prevent silting of waterways or flooding damage to the property and adjacent property.

3.12  	Erosion Control

Contractor shall prevent erosion of soil on the site and adjacent property resulting from his construction activities.  Effective measures shall be initiated prior to the commencement of clearing, grading, excavation, or other operations which will disturb the natural protection.

CONTRACTOR shall use controls found in "Environmental Criteria Manual" or developed from successful techniques elsewhere as approved by E/A.  Siltation and/or sedimentation controls shall include dams, berms, and dikes as recommended in the "Environmental Criteria Manual".

3.13   Pollution Control

Contractor shall prevent the pollution of drains and watercourses by sanitary wastes, sediment, debris and the substances resulting from construction activities.  No sanitary wastes will be permitted to enter any drain or watercourse. No sediment, debris or other substance will be permitted to enter sanitary sewers and reasonable measures shall be taken by CONTRACTOR to prevent such materials from entering any drain or watercourse.

Contractor shall observe the rules and regulations of the State of Texas and agencies of the U.S. Government prohibiting the pollution of any lake, stream, river, or wetland by the dumping of any refuse, rubbish, dredge material, or debris therein.

Contractor is specifically cautioned that disposal of materials into any water of the State must conform to the requirements of the Texas Commission on Environmental Quality (TCEQ), and any applicable permit from the U.S. Army Corps of Engineers.

3.14   Noise Control

CONTRACTOR shall comply with the City of Austin’s Noise Ordinance.  CONTRACTOR shall take reasonable measures to avoid unnecessary noise.  Such measures shall be appropriate for the normal ambient sound level in the area during working hours.  All construction machinery and vehicles shall be equipped with practical sound-muffling devices, and operated in a manner to cause the least noise consistent with efficient performance of the Work.

3.15   CIP Sign

CONTRACTOR shall erect install and maintain CIP signs as specified.  Signs shall be constructed in accordance with City Standard Specification Item No. 802S “Project Signs,”  as indicated on the Drawings.

3.16   Fences

All existing fences affected by the Work shall be maintained by the CONTRACTOR until completion of the Work.  Fences which interfere with construction operations shall not be relocated or dismantled until written permission is obtained from the owner of the fence, and the period the fence may be left relocated or dismantled has been agreed upon.  Where fences must be maintained across any construction easement, adequate gates shall be installed.  Gates shall be kept closed and locked at all times when not in use.

Upon completion of the Work across any tract of land, CONTRACTOR shall restore all fences to preconstruction, or to a better, condition and to their preconstruction location.

3.17   Mail Boxes

Contractor shall remove, reset temporarily, and relocate permanently all mail boxes that are within construction site limits conforming to requirements of United States Postal Service.  Mailboxes shall not be laid on the ground, but shall be temporarily reset the same day as removed.  Payment for removing and resetting of mail boxes will not be paid for directly, but will be considered subsidiary to the various Bid items.  Any damage to mail boxes or posts shall be the responsibility of the Contractor.

3.18   Emergency Facilities

Free access shall be maintained at all times to fire lanes and emergency and utility control facilities such as fire hydrants, fire alarm boxes, police call boxes, and utility valves, manholes, junction boxes, etc.  In the event that it is necessary to make one of these facilities temporarily inaccessible, Contractor shall obtain approval of such action and schedule of Work from the Owner.  CONTRACTOR shall also provide at least 24 hours prior notice to the Fire Department, Police Department, and City Department governing the affected utility.  The same Department(s) shall be promptly notified by the Contractor when such facilities are placed back in unobstructed service.

3.19   Notification of Owners

Unless otherwise indicated, the OWNER will notify property owners abutting the right-of-way of impending construction.  The CONTRACTOR shall exercise diplomacy and tact with individual property owners.

3.20   Maintenance of Traffic

Contractor shall conduct his Work to interfere as little as possible with public travel, whether vehicular or pedestrian.  Whenever it is necessary to cross, obstruct, or close roads, driveways, and walks whether public or private, the Contractor shall provide and maintain suitable safe bridges, detours or other temporary measures to accommodate public and private travel, and shall provide reasonable notice to owners of private drives before interfering with them.  Such maintenance of traffic will not be required when Contractor has obtained written permission from the owner and the tenant of the private property, or from the authority having jurisdiction over public property involved, to obstruct traffic at the designated point.  A copy of the initial written permission shall be provided to the Owner’s Representative.

Safety and conveyance of traffic shall be regarded as prime importance.  Unless otherwise directed, all portions of streets associated with this Project shall be kept open and provided a dust free, smooth and comfortable ride to traffic.  It shall be the responsibility of the Contractor to ensure that two-way traffic may safely bypass the construction site and that access is provided to abutting private property.  In making open cut street crossings, the Contractor shall not block more than one-half of the street at one time without approval of the Owner.  Whenever possible, Contractor shall widen the shoulder on the opposite side to facilitate traffic control.  Temporary surfacing shall be provided as necessary on shoulders.

Prior to beginning Work, CONTRACTOR shall designate, in writing, a competent person who will be responsible and available on the Project site, or in the immediate area, to ensure compliance with the traffic control plan.  CONTRACTOR shall provide documentation to demonstrate the sufficient training in Traffic Control for his competent person. Owner will designate a qualified person to observe implementation and who will have authority to assure compliance with the traffic control plan.

The CONTRACTOR shall perform the necessary cleanup and finishing immediately after all or a portion of the Work is completed.  When the Work includes paving operations, the entire site shall be kept clean to facilitate placement of required traffic control devices.  Temporary and permanent striping lay-out shall be approved by the Transportation Department prior to placement, when included in the Work.

- Detours

Where indicated on the traffic control plan CONTRACTOR shall erect and maintain detours around construction activities.   Should CONTRACTOR desire to propose a detour, not already included in the traffic control plan, it shall be his responsibility to prepare a revised traffic control plan showing the detour, and obtain approval of the revised traffic control plan from the Transportation Department, prior to implementation of the detour.  The Transportation Department has final authority as to the acceptability of any proposed revisions to the traffic control plan.  The CONTRACTOR shall bear all costs for revising the traffic control plan and for maintaining the proposed detour.

- Barricades and lights

CONTRACTOR shall place and maintain in good condition, standard barricades at each end of the Project and at other locations where traffic is rerouted or blocked from using regular traffic lanes.  Barricades and warning signs shall be in accordance with the Texas Manual on Uniform Traffic Control Devices (MUTCD) and City of Austin Standard Specification Item No. 803S, “Barricades, Signs and Traffic Handling”.  Signs, barricades, and warning devices informing the public of construction features will be placed and maintained by the CONTRACTOR, who shall be solely responsible for their maintenance.  The decision to use a particular device at a particular location as indicated in the traffic control plan or as determined by the CONTRACTOR, shall be the sole responsibility of the CONTRACTOR.

All open trenches and other excavations shall have suitable barricades, signs, and lights to provide adequate protection to the public.  Obstructions, such as material piles and equipment shall be provided with similar warning signs and lights.

All barricades and obstructions shall be illuminated with warning lights from sunset to sunrise.  Material storage and conduct of the Work on, or along side, public streets and highways shall cause a minimum obstruction and inconvenience of the traveling public.

3.21	 Required Job Site Postings and Notices

CONTRACTOR shall post the following postings and notices in English and Spanish at one or more conspicuous locations on the job site.  In the case of Projects with multiple sites, the notices and postings must be displayed at each site.  In the case of Projects that do not have a job shack or other temporary facility on the site, CONTRACTOR shall post all notices on a temporary bulletin board. Other special conditions are noted below.

| Required for all Projects |  |
| --- | --- |
| Poster | Available at: |
| Baseline Schedule for Project identifying when all subcontractors will be used | N/A (as required under Section 00700, paragraph 2.4.2.1.) |
| Wage Rates as required under Section 00830. | Section 00830BC and/or Section 008300HH |
| City of Austin Wage Contact posters | Provided at Pre-Construction meeting (English and Spanish) |
| City of Austin Equal Employment Opportunity posters | http://austintexas.gov/department/wage-compliance (English and Spanish) |
| Texas Commission on Environmental Quality “Construction Site Notice” form, if applicable, as required or the required TPDES information | http://www.tceq.state.tx.us/assets/public/permitting/waterquality/attachments/stormwater/txr152d2.pdf (Option 1 – as required under Section 00810, 6.7.4.2 N/A (Option 2 – as required under Section 00810, 2.6.7.4.3) |
| OSHA poster “Job Safety and Health: It's the Law” | http://www.osha.gov/Publications/osha3165.pdf (English) http://www.osha.gov/Publications/osha3167.pdf (Spanish) |
| City of Austin Rest Break Ordinance Signs | http://austintexas.gov/department/wage-compliance (English and Spanish) As required to be posted in English and Spanish under Ordinance No. 20100729-047 |
| Texas Payday Law Poster | http://www.twc.state.tx.us/ui/lablaw/ll10.pdf (English) http://www.twc.state.tx.us/ui/lablaw/ll10s.pdf (Spanish) |
| Texas Workers Compensation notice that the employer does or does not carry Workers Compensation insurance | Does not carry Workers Compensation Insurance: http://www.tdi.state.tx.us/forms/dwc/notice5.pdf (English) http://www.tdi.state.tx.us/forms/dwc/notice5s.pdf (Spanish) Does carry Workers Compensation Insurance: http://www.tdi.state.tx.us/forms/dwc/notice6.pdf (English) http://www.tdi.state.tx.us/forms/dwc/notice6s.pdf (Spanish) |
| TWC Employer’s Notification of the Ombudsman Program to Employees | http://www.oiec.state.tx.us/documents/Employer_Notice_of_O.pdf (both versions) |
| DOL – The Uniformed Services Employment and Reemployment Rights Act (USERRA) | http://www.dol.gov/vets/programs/userra/USERRA_Private.pdf |
| EEOC Equal Employment Act and the Americans with Disabilities Act (ADA) | http://www.dol.gov/ofccp/regs/compliance/posters/pdf/eeopost.pdf (English) http://www.dol.gov/ofccp/regs/compliance/posters/pdf/eeosp.pdf (Spanish) |
| Fair Labor Standards Act (FLSA) Minimum Wage Poster | http://www.dol.gov/whd/regs/compliance/posters/flsa.htm (English) http://www.dol.gov/whd/regs/compliance/posters/flsaspan.htm (Spanish) |
| If applicable: Employee Rights for Workers with Disabilities/Special Minimum Wage Poster – Employment Standards | http://www.dol.gov/whd/regs/compliance/posters/disabc.pdf http://www.dol.gov/whd/regs/compliance/posters/disabspanc3p.pdf |
| “Your Rights Under the Family and Medical Leave Act (FLMA)” | http://www.dol.gov/whd/regs/compliance/posters/fmlaen.pdf (English) http://www.dol.gov/whd/regs/compliance/posters/fmlasp.pdf (Spanish) |
| Title VI Rights Poster | http://austintexas.gov/department/wage-compliance |
| Additional Postings Required for Federally Funded Projects |  |
| “Employee Rights Under the Davis-Bacon Act” | http://www.dol.gov/whd/regs/compliance/posters/fedprojc.pdf (English) http://www.dol.gov/whd/regs/compliance/posters/davispan.pdf (Spanish) |
| Applies to USDOT/FHWA funded projects: “Notice of False Statements Concerning Highway Projects” | http://www.mdt.mt.gov/publications/docs/forms/dbe/eeo_board/false_statements.pdf (as required in Section 00810A Standard Federal-Aid Assurances) |
| Applies to USDOT/FHWA funded projects; “Contractors EEO Policy” | N/A (as required in Section 00810A Standard Federal-Aid Assurances) |

END